Meteorological Data Profiles. The meteorological data of Pina-anan River from the telemetry station of Tagoloan HPP which is located at Impasugong Bukidnon with latitude 8.328403° and longitude 125.017313°. The profiles in Figure A and Figure B show the 6-month precipitation and water level data from January 27, 2020 to June 30, 2020 that is based on a 24-hour monitoring of rainfall data (in millimeter unit) and water level data (in meter unit). The precipitation data (Fig. A) revealed high readings of 25.2mm and 22.5mm on May 30, 2020 at 3:00PM and March 6, 2020 at 6:00 PM, respectively. Consequently, the highest water level (Fig. B) record is 3.71m on June 18, 2020 with a mode value of 1.5m.

Figure A. Pina-anan River Precipitation Profile from January 2020 to June 2020

Figure B. Pina-anan River Water Level Profile (January 2020 to June 2020)

The combined plot of rainfall and water level profile is shown in Figure C. This profile revealed two high records of data values, namely: (1) March 6, 2020 with precipitation of 22.8mm and water level of 2.227m; and (2) May 30, 2020 with 25.2mm of precipitation and a water level of 1.70m. Moreover, there was a series of rainfall from June 9 to June 19 period with accumulated precipitation value of 16.6mm on June 11 and accumulated water level of 3.56m on June 18. Finally, results revealed raining season from June 6 – June 29 with an average rainfall value of 0.44mm.

Figure C. Pinan-anan River Water Level and Precipitation Plot (January 2020 to June 2020)

Metrological Data Analysis. Figure D show the Pinan-anan River Precipitation Profile. The data is an hourly profile January 1, 2020 – June 30, 2020 of Pina-anan River gauging from telemetry station of Tagoloan HPP located at Impasugong Bukidnon (8.328403°, 125.017313°). There are erroneous rainfall data from May 18, 2021 to June 30, 2021 with highest rainfall of 1.15 mm and least rainfall value of 0.008 mm. The rainfall Data starting on May 18 to June 30 is insignificant for the Hyeto-Hydrograph since it formulates of an inaccurate trend for the Rainfall and Water Level. On the other hand, data revealed that the highest rainfall on May 10, 2021 (1:00pm) with a precipitation value of 33.20 mm and April 14, 2021 (6:00PM) with a precipitation value of 20.4 mm

Figure D. Pinan-anan River Precipitation Plot (January 2021 to July 2021)

Figure E. Pinan-anan Water Level Plot (March 2021 to June 2021)

Figure E show the hourly water level data of Pina-anan River from telemetry station in Tagoloan HPP located at Impasugong Bukidnon (8.328403°, 125.017313°). The hourly data is recorded from March 1, 2021 to June 30, 2021 with the highest water level (April 23) value of 3.56 m and a mode value of 1.7026m.

Figure F There are erroneous rainfall data from May 18, 2021 to June 30, 2021 with highest rainfall of 1.15 mm and lowest rainfall of 0.008mm. Based from observation, the data record from May 18, 2021 does not coincide with the trend i.e. rainfall data and water level. The highest precipitation were found last March 10, 2021 at 33.20mm with a water level measurement of 1.965m and last April 14, 2021 at 20.4mm with a water level measurement of 2.20m. Also, series of rainfall from April 14 to April 20 with a highest rainfall value of 20.4mm generate the highest water level of the four months with a value of 3.56m on April 23, 2021.

Figure F. Pinan-anan River Water Level and Precipitation Plot

Data Correlation

Figure G. Pinan-anan River Tagoloan Rating Curve (2015 to 2020)

The graph in Figure G shows the previous data of the relationship between the stage and Flow for 2015 -2020 which generate an rating curve for the last 5 years (2015-2020) with (R = 0.7211) which represents of the proportion variance between relationship of the water level and flow. The graph indicates a vary changes of the water level and flow every year whereas flow measurement of year 2015 & 2016 shows an outlier of the curve for the past 5 years.

Figure H. Pinan-anan River Tagoloan Rating Curve (2018 to June 2020)

Feb 11,2020 10:49 AM & 10:49 AM

Feb 4,2020 10:50 AM & 11:03 AM

Figure I. Pinan-anan River Tagoloan Rating Curve (2018 to June 2020)

-The highest correlation between the stage and flow was found between the year 2018-2019 (R² = 0.49). generally, has been considered to be the most representative data for estimation of Tagoloan Hydrograph for the preceding months.

- it shows a variance of 88% of its correlation for the last three years of water level & flow measurement.

-Due to pandemic, a flow measurement was taken during Feb 4 and Feb 11 for the year 2020 whereas only two sample where formulated during the year 2020

- there is a restrictions of the accuracy for the formulation of the rating curve due to the limitation of the data particularly in the flow measurements especially a series of rainfall event had been observed for the preceding months.

Rating Curve is expressed as an equation of the following form in the Agency’s hydrometric archive system;

Q = C (h + a) β

where C, a and β, are coefficients that do have a physical significance as they were originally derived from hydraulic theory. The coefficient C increases as river cross-sectional area and slope increase, but decreases as roughness increases. It can be shown that for simple geometries the β coefficient is constant where β coefficient should remain unchanged. a is related to the elevation of the bed, and should also remain unchanged whereas the rating can be simplified into a power type of equation of

Q =aSb

where a and b are parameters that are fit using a power regression, S is stage height in meters and Q is the discharge in m³/s.

Taking logarithms of the power type equation results in a straight line relationship of the form.

log(Q)  = log(a) + b log(S)

Fit a linear line to the log-log data

The sets of discharge (Q) and the effective stage S are plotted on the double log scale, they will represent a straight line. Coefficients A and B of the straight line fit are functions of a and b. Since values of a and b can vary at different depths owing to changes in physical characteristics (effective roughness and geometry) at different depths, one or more straight lines will fit the data on double log plot.

log(Q) = 0.3813 + 3.6653 log(S)

Extract the coefficients

a = 100.3813 = 2.406024;  b = 3.6653

Therefore, the rating curve is expressed into a power equation of

Q =aSb

Q(1s) =2.406024  (S[m])3.6653

-correlation of the observed flow on 2016-2020 and simulated flow with has an average error of 0.121 and RMSE of 1.90

- it directs that the equation can be somehow used to simulate the hydrograph based from the observed water level from the telemetry. Since it has a limitation of the data and based from the there is a rapid of changes of its coefficients and parameters as the time goes by

Rating Curve

In order to derive a rating curve for the preceding months, despite of the two sample flow measurements has been flow measured. A plot of the water level WL observed at the gauge versus the simulated flow base from the derive power equation for the following months.

- it shows that the simulated hydrograph base from the derive equation is proportional to the water level from the given time

-The highest flow levels in simulation were observed in June 18,2020 with a flowrate of 293.36 cms and has also the highest water level of 3.71m  , the lowest water levels were observed on  May 3,2020 & May 8.2020 which has flowrate of 7.12cms and a stage of 1.34 m

Result and discussion

MARCH 2020

-The highest flow levels in simulation were observed in March 6,2020 with a flowrate of 22.8 cms and has highest rainfall value of 22.8mm of the month

- highest flow rate: 47.14 cms with a stage of 2.25m

- lowest flow rate: 7.59 with a stage of 1.3618

- average flow of the month: 11.84 cms

- mode value for water level. 1.516m

APRIL 2020

- The highest flow rate simulation were observed in April 20,2020 with a flowrate of 26.16 cms and has highest rainfall value of 6mm on April 18

- highest flow rate: 26.16  cms with a stage of 1.917m

- lowest flow rate: 7.59 cms with a stage of 1.3618 m

- average flow of the month: 9.98 cms

- mode value for water level. 1.4638

MAY 2020

-The highest flow rate  simulation were observed in May 12,2020 with a flowrate of 70.62 cms and has highest rainfall value of 25.2 mm on May 30

- highest flow rate: 70.62  cms with a stage of 2.51m

- lowest flow rate: 7.12 cms with a stage of 1.344 m

- average flow of the month: 13.76 cms

- mode value for water level. 1.4638

JUNE 2020

The highest flow rate simulation were observed in June 11,2020 with a flowrate of 293.36 cms and has highest rainfall value of 16.6 mm on June 11

- highest flow rate: 293.36  cms with a stage of 3.7088m

- lowest flow rate: 9.16 cms with a stage of 1.4399 m

- average flow of the month: 29.72 cms

- mode value for water level. 1.5592

- lowest flow rates were observed on the Month of May with a flow rate value of 7.12 cms and Highest flow rate value were observed on the month of June with a flowrate value 293.6 cms.

- Base from the meteorological data the month of July have an accumulated rainfall value of 287.40mm and an average value of rainfall of 0.09 which shows that the month of June is describe as rainy season from the 4 months

- The rating curve express into a polynomial trendline to shows the differences of the distinct behavior of the rating curve for the 4 months

- It indicates there is prompt changes of characteristics that happen within the area. It shows that there is an increase of base flow for every month whereas 1.28% flow rate from the month of May to June.

- It shows the Month of June has the highest grade of its curve which emphasize a volatile flowrate within the month. And the month of April has less series of rainfall that happen in the 4 months.

MARCH 2021(c/o Gif)

-The highest flow levels in simulation were observed in March 14, 2021 with a flowrate of 68.2 cms and has highest rainfall value of 16.4 mm on March 10, 2021

- highest flow rate: 68.2  cms with a stage of 2.49m

- lowest flow rate: 12.96 with a stage of 1.583

- average flow of the month: 18.98 cms

- mode value for water level. 1.7026

APRIL 2021

-The highest flow levels in simulation were observed in April 23, 2021 with a flowrate of 253.91 cms and has highest rainfall value of 20.4 mm on April 14,2021

- highest flow rate: 253.91  cms with a stage of 3.56m

- lowest flow rate: 10.63 with a stage of 1.50

- average flow of the month: 23.61 cms

- mode value for water level. 1.83 m

MAY 2021

-The highest flow levels in simulation were observed in May 26, 2021 with a flowrate of 181.4 cms and has highest rainfall value of 33.2 mm on May 10,2021

- highest flow rate: 181.84  cms with a stage of 3.25m

- lowest flow rate: 13.69 cms with a stage of 1.61 m

- average flow of the month: 28.6 cms

- mode value for water level. 1.70 m

JUNE 2021

-The highest flow levels in simulation were observed in June 22 2021 with a flowrate of 196.94 cms and followed by a flowrate of 131.64 cms on June 1, 2021

- highest flow rate: 196.94  cms with a stage of 3.33m

- lowest flow rate: 23.85 cms with a stage of 1.87 m

- average flow of the month: 41.85 cms

- mode value for water level. 2.01 m

- lowest flow rates were observed on the Month of April with a flow rate value of 10.63 cms and Highest flow rate value were observed on the month of April with a flowrate value 253.91 cms.

- mode value for water level (stage) for the 4 months is 1.70 which comprehends a base flow of 31.256 cms

- The rating curve express into a polynomial trendline to shows the differences of the distinct behavior of the rating curve for the 4 months

- It indicates there is prompt changes of characteristics that happen within the area. It shows that there is an transition of base flow for every month whereas above 1.70 m flow rate for  the month of March & April and above 1.80 for the month of May June which there is an 0.10 cms  fluctuations of base flow in every month .

- It shows the Month of June has the highest grade of its curve which emphasize a volatile flowrate within the month. And the month of April & March have less series of rainfall that happen in the 4 months.

- Increase of 44.67% on its highest flow rate (21.06 cms added)

- Increase of 71.15% on its lowest flow rate: (5.37 cms added)

- Increase of 60.30% on its Average flow month: (7.14 cms added)

- Increase of 12.31% on its Average flow month: (0.187 m added)

- Increase of 870.6% on its highest flow rate (227.75 cms added)

- Increase of 40.05% on its lowest flow rate: (3.04 cms added)

- Increase of 136.57% on its Average flow month: (13.63 cms added)

- Increase of 25.01% on its Average flow month: (0.366 m added)

- Increase of 157.49% on its highest flow rate (111.22 cms added)

- Increase of 92.28% on its lowest flow rate: (6.57 cms added)

- Increase of 107.84% on its Average flow month: (14.84 cms added)

- Increase of 16.14% on its Average flow month: (0.2362 m added)

- Decrease of 32.87% on its highest flow rate (96.42 cms deducted)

- Increase of 160.37% on its lowest flow rate: (14.69 cms added)

- Increase of 40.81% on its Average flow month: (12.13 cms added)

- Increase of 28.91% on its Average flow month: (0.451m added)

Limitations & Recommendations

-Flow measurements are so much limited due to pandemic.

-additional of RG for meteorological measurement particularly within the basin and nearly the area.

- Provide/Gathered more sample of flow measurement in order to establish a precise rating curve (for more accurate flow measurement, a use of Digital flow meter to determine an minutes/hourly/daily velocity rate)

-establish a cross-section survey to emphasize the left and right banks of the cross-section which can indicates for a bank-full scenario. And integrated the cross-section profile with the establishment of water surface level on the given time from the surveyed.

- Establish an Hydrologic Modelling which makes an accurate measurement of flow and good information for the assessments for the observed data with respect to long terms trends.

- Erroneous of data which affect the analysis of the rating curve. Time to time, data must be evaluated if it is substantial to accurate figures of data and   device must be evaluate if it still working or calibrated.